Altium DXP Mod Player Example

Mod is a music player scheme allowing for playing 8 bit audio samples with a sequencer. This project plays MOD files (*.mod) containing both the samples and the sequencer information, which are available on the Web from many sources. The project folder includes a Mod folder with several Mod files that can be played.

Information on the Mod standard is available at http://www.wotsit.org/search.asp?s=mod%20file

There are versions of the project that will run on the NB1 Nanoboard equipped with a Daughterboard with SRAM, and on the EB1/EB2 Nexar Evaluation Boards. This is the case because the Evaluation Boards and the Nanoboard have different audio output resources.

The project generates four channels of 8 bit mod audio using hardware. The Mod hardware obtains audio samples, adjusts the pitch and duration of samples and delivers the resulting audio to the TSK3000 processor. The TSK3000 manages the operation of the Mod hardware, downloading Mod files through a serial port and digitally processing the audio channels in an interrupt running at 22050 Hz. The software provides a 16 bit 4 channel stereo mixer with reverb. The reverb unit has two delays with feedback and a selectable IIR low pass filter. On the NB1 the stereo audio channels are summed to produce an 8 bit mono output which is driven to the NB1 MAX1104 SPI DAC. The EB version has stereo 12 bit Delta Sigma DACs which drive the EB stereo audio system.

Mixer settings are fixed but can be changed by editing function AUDIOMIXER_Init() in AudioMixer.c

The Mod files are sent to the Mod Player via the PC serial port. Connect the target device EB or NB1 serial port to COM1 on the PC.

When developing the Mod Player a freeware terminal program was obtained from http://hp.vector.co.jp/authors/VA002416/teraterm.html. Obtain and install Tera Term Pro. The required serial port settings are :

Setup>Serial Port….>(Baud rate : 115200, 1 Stop Bit, 8 data bits, No Parity, No Flow Control, zero delays)

Mod Files are sent to the Mod Player with File>Send File. In the Send File file selection dialog the Binary option must be checked before the file is sent.

Running the Project

Connect the EB or NB1 target device to the PC. Connect a serial cable from the target to the PC. Power up the target EB or NB1. Run Tera Term Pro.

The target EB or NB1 dip switches control aspects of the audio output.

Switch			Function (ON)

- Enable Mod Channel 0

- Enable Mod Channel 1

- Enable Mod Channel 2

- Enable Mod Channel 3

- Enable Reverb

- Enable Reverb Low Pass

Initially set all dip switches to ON.

In DXP Open and Build the Project.

The Tera Term Pro terminal should indicate you should download a Mod file. Use File>Send File...  to select and transfer a Mod file to the Mod Player.

You are then prompted to press ‘p’ to play the Mod File. Use headphones or powered speakers to hear the resulting Mod File.

Configuration

See Mod Player top schematic for configuration data relating to FPGA targets.