Simple Network Application

About:

The Simple Network Application provides an example use of the files “CANDriver.asm” and “SimpleNetworkProtocol.asm” which implement low-level driver code for the CAN controller, and a simple protocol for network communication. The protocol supports just 2 basic function – Getting and Setting network variable values. A network variable is an application defined value on an embedded device. To get or set a network variable, the client of the protocol need only specify the target node ID (1 – 31 for individual nodes, 0 for broadcast to all nodes) and the address of the variable (as well as its new value in the case of the 'Set' operation).

In this application, setting the variable at address 0x01 will turn the LEDs on the nanoboard in a pattern which depends on the value sent. Getting the value from address 0x02 will retrieve the value on the Digital I/O soft device.

Usage:

This project is for use with 2 or more nanoboards connected via the CAN port. Each nanoboard should be assigned a unique Node ID (though if the boards are to be targets of “Set” operations only and they are meant to receive all the same messages they can, of course, have the same Node ID). To assign a Node ID, set the dipswitches on the nanoboard to the desired value, using the switches 1-5. This will be the actual Node ID (in binary format). For example, to set a node to be ID 1, flick the 1st switch to the on position. To make Node ID 0x06, flick switches 2 and 3 only to the on position.

When the node is reset, it will initially display its Node ID in hexadecimal format as set on the dipswitches.

- Pressing any number on the keypad will set that value as the target node. Note that this means that this example application can only address nodes up to ID 9, even though the network protocol can handle higher node IDs.

- Pressing the 'A' key will toggle between the two possible network protocol operations 'GET' and 'SET'. The current mode will thereafter be displayed, along with the ID current target node. The target node is the address where the 'GET' or 'SET' message will be sent.

- Pressing any other letter in 'SET' mode will send that value (ie., 0x0b, 0x0c,0x0d, 0x0e, 0x0f) to the target node.

- Pressing any other letter in 'GET' mode will initiate the 'GET' operation. The target node will be queried for the value of its Network Variable 0x02, which returns the value on the target board's Digital I/O soft device.